MSAM Excel Library Functions

Note: double and string indicate the return type and argument types for functions.

Log to Linear Conversions

double dBuV_to_Volts(double dBuV) - Convert dBµV to Volts

double Volts_to_dBuV(double Volts) - Convert Volts to dBµV

double dBV_to_dBuV(double dBV) - Convert dBV to dBµV

double dBuV_to_dBV(double dBuV) - Convert dBµV to dBV

double dBV_to_Volts(double dBV) - Convert dBV to Volts

double Volts_to_dBV(double Volts) - Convert Volts_to_dBV

Log to Linear Power Conversions

double dBm_to_dBW(double dBm) - Convert dBm to dBW

double dBW_to_dBm(double dBW) - Convert dBW to dBm

double dBm_to_Watts(double dBm) - Convert dBm to Watts

double Watts_to_dBm(double Watts) - Convert Watts to dBm

double dBW_to_Watts(double dBW) - Convert dBW to Watts

double Watts_to_dBW(double Watts) - Convert Watts to dBW

Log to Linear Current Conversions

double dBuA_to_uA(double dBuA) - Convert dBµA to µA

double uA_to_dBuA(double uA) - Convert µA to dBµA

double dBA_to_A(double dBA) - Convert dBA to A

double A_to_dBA(double A) - Convert A to dBA

double dBA_to_dBuA(double dBA) - Convert dBA to dBµA

double dBuA_to_dBA(double dBuA) - Convert dBµA to dBA

Field Strength Power Density

double dBuVpm_to_Vpm(double dBuVpm) - Convert dBµV/m to V/m

double Vpm_to_dBuVpm(double Vpm) - Convert V/m to dBµV/m

double dBuVpm_to_dBmWpm2(double dBuVpm) - Convert dBµV/m to dBmW/m2

double dBmWpm2_to_dBuVpm(double dBmWpm2) - Convert dBmW/m2 to dBµV/m

double dBuApm_to_dBuVpm(double dBuApm) - Convert dBµA/m to dBµV/m

double dBuVpm_to_dBuApm(double dBuVpm) - Convert dBµV/m to dBµA/m

double dBuApm_to_dBpT(double dBuApm) - Convert dBµA/m to dB/T

double dBpT_to_dBuApm(double dBpT) - Convert dB/T to dBuA/m

double Wpm2_to_Vpm(double Wpm2) - Convert W/m2 to V/m

double Vpm_to_Wpm2(double Vpm) - Convert V/m to W/m2

double uT_to_Apm(double uT) - Convert µT to A/m

double Apm_to_uT(double Apm) - Convert A/m to µT

Distance Conversion

double Feet_to_Meters(double Feet) - Convert Feet to Meters

double Meters_to_Feet(double Meters) - Convert Meters to Feet

double Miles_to_Kilometers(double Miles) - Convert Miles_to_Kilometers

double Kilometeres_to_Miles([double Kilometers) - Convert Kilometers to Miles

double Feet_to_Miles(double Feet) - Convert Feet to Miles

double Miles_to_Feet(double Miles) - Convert Miles to Feet

double Meters_to_Kilometers(double Meters) - Convert Meters to Kilometers

double Kilometers_to_Meters(double Kilometers) - Convert Kilometers to Meters

Temperature Conversion

double Fahrenheit_to_Celsius(double Fahrenheit) - Convert Fahrenheit to Celsius

double Celsius_to_Fahrenheit(double Celsius) - Convert Celsius to Fahrenheit

double Celsius_to_Kelvin(double Celsius) - Convert Celsius to Kelvin

double Kelvin_to_Celsius(double Kelvin) - Convert Kelvin to Celsius

double Fahrenheit_to_Kelvin(double Fahrenheit) - Convert Fahrenheit to Kelvin

double Kelvin_to_Fahrenheit(double Kelvin) - Convert Kelvin to Fahrenheit

Miscellaneous Conversion

double gpm3_to_hPa(double gpm3, double Temp_Kelvin) - Convert g/m3 to hPa

Geographic

double DistKm(double Latitude1, double Longitude1, double Latitude2,double Longitude2) - Distance (Km) between two coordinates using WGS84 Datum

double Bearing(double Latitude1,double Longitude1, double Latitude2, double Longitude2) - Bearing (DecDeg) form (Lat1,Lon1) to (Lat2,Lon2) using WGS84 Datum

double GetLatitude(double Latitude, double Longitude, double DistKm, double Bearing) - Get Latitude From Coordinate/Distance/Bearing

double GetLongitude(double Latitude, double Longitude, double DistKm, double Bearing) - Get Longitude From Coordinate/Distance/Bearing

double Delta_Bearing(double Bearing1, double Bearing2) - Angle in degrees between two bearings

double DMS2DecDeg(string dms) - Degrees Minutes Seconds string to Decimal Degrees

string DecimalDeg2DMSLat(double decdeg) - Latitude Decimal Degrees to Degrees, Minutes, Seconds, N/S

string DecimalDeg2DMSLon(double decdeg) - Longitude Decimal Degrees to Degrees, Minutes, Seconds, E/W

double TerrainHeight_m(double Latitude, double Longitude, int TerHandler, string TerDirectory) - Terrain Height (m), NaN if not in database.

P.835 Reference Standard Atmospheres

double P835WaterVaporPressure_hPa(double Height_Km)

double P835WaterVapoDensity_gpm3(double Height_Km)

double P835Temperature_Kelvin(double Height_Km)

Propagation

double RadioHorizon_Km(double AntennaHeight_m)

double FDR_Stairs_3_20_40_60_dB(double Tx_3dB_BW, double Tx_20dB_BW,double Tx_40dB_BW, double Tx_60dB_BW, double Rx_3dB_BW, double Rx_20dB_BW,double Rx_40dB_BW, double Rx_60dB_BW,double Tx_Frq_MHz, double Rx_Frq_MHz)

double SatAz(double EarthStationLatDeg, double EarthStationLonDeg, double SatelliteLatDeg, double SatelliteLonDeg, double SatelliteAltitudeKm, int RetVal)

double BmWToGain_dB(double Beamwidth_Deg,double Efficiency)

double StatGain(double OffAxis_Deg, double MainBeamGain_dB)

double StatGainWolfGain(double OffAxis_Deg, double MainBeamGain_dB)

double FreeSpace(double Dist_Km, double Freq_MHz)

double ITMAreaCR(double TxHtm, double RxHtm, int TxSC, int RxSC, double Dist_Km, double DeltaHm, int RadClim, double SurfRef,double FreqMHz, int pol, double permittivity, double conductivity, int mdvar, double confidence, double reliability)

double ITMAreaTLS(double TxHtm, double RxHtm, int TxSC, int RxSC, double Dist_Km, double DeltaHm, int RadClim, double SurfRef, double FreqMHz, int pol, double permittivity, double conductivity, int mdvar, double timepct, double locationpct, double situationpct)

double ITMPointToPointCR(double TxHtm, double RxHtm, double TxLat, double TxLon, double RxLat, double RxLon, double Dist_Km, double DeltaHm, int RadClim, double SurfRef, double FreqMHz, int pol, double permittivity, double conductivity, int mdvar, double confidence, double reliability, int TerHandler, string TerDirectory)

double ITMPointToPointTLS(double TxHtm, double RxHtm, double TxLat, double TxLon, double RxLat, double RxLon, int RadClim, double SurfRef, double FreqMHz, int pol, double permittivity, double conductivity, int mdvar, double timepct, double locationpct, double situation, int TerHandler, string TerDirectory)

double eHata(double TxLat, double TxLon, double RxLat, double RxLon, double HgtBase_m, double HgtMobile_m, double FreqMHz, int CityDensityenviro_code, double reliabilitypct, int TerHandler, string TerDirectory)

double P676Annex1 (double Freq_GHz, double Pressure_hPa, double WatVap_gpm3, double Temp_C, double DistKm)

double P676_SpecificAttenuation_dB(double Freq_GHz, double Temp_K, double WaterVapuorPartialPressure_hPa, double DryAirPressure_hPa, double Dist_Km)

double P528_5(double Dist_Km, double hgt1_m, double hgt2_m, double Freq_MHz, int PolarizationHV, double TimePct)

object P528_5_Details(double Dist_Km, double hgt1_m, double hgt2_m, double Freq_MHz, int PolarizationHV, double TimePct, int Result)

string IntermodHarmonics(double[] TxFreqMHz, double[] RxFreqMHz, double[] RxBWMHz)

string IntermodTwoSignalMix(double[] TxFreqMHz, double[] RxFreqMHz, double[] RxBWMHz, int Option)

string IntermodThreeSignalMix(double[] TxFreqMHz, double[] RxFreqMHz, double[] RxBWMHz, int Option)

double OkumHata529(double FreqMHz, double AntennaBaseHt_m, double AntennaMobileHt_m, double Dist_Km, int AreaType, int CitySize, double TimePct, double LocPct)

double Cost231(double FreqMHz, double AntennaBaseHt_m, double AntennaMobileHt_m, double Dist_Km, int AreaType, int CitySize, double TimePct, double LocPct)

double OkumHataDavidson(double FreqMHz, double AntennaBaseHt_m, double AntennaMobileHt_m, double Dist_Km, int AreaType, int CitySize, double TimePct, double LocPct)

double OkumHata529FieldStrength(double FreqMHz, double AntennaBaseHt_m, double AntennaMobileHt_m, double Dist_Km, int CitySize, int AreaType, double TimePct, double LocPct,

double Power_Watts)

double Cost231FieldStrength(double FreqMHz, double AntennaBaseHt_m, double AntennaMobileHt_m, double Dist_Km, int CitySize, int AreaType, double TimePct, double LocPct, double Power_Watts)

double OkumHataDavidsonFieldStrength(double FreqMHz, double AntennaBaseHt_m, double AntennaMobileHt_m, double Dist_Km, int CitySize, int AreaType, double TimePct, double LocPct, double Power_Watts)